1 Portable ladders (not stepladders) should always be placed at the correct angle, which is around 75 degrees or roughly one metre out for every four metres up (the 1 in 4 rule).

83

Cleaning - No.42 Work at Height - Ladders (List additional hazards, risks and controls particular to your school using the blank template no.55)

| Hazards | Is the hazard present? Y/N | What is the risk? | Risk rating H = High M = Medium L = Low | Controls (When all controls are in place risk will be reduced) | Is this control in place? | Action/to do list/outstanding controls *Risk rating applies to outstanding controls outlined in this column | Person responsible | Signature and date when action completed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unsafe use of ladders and step ladders |  | Falls from height | H | Ladders are only used for short duration work (less than 30 minutes) and where the risk of injury is low |  |  |  |  |
| Unsafe storage of ladders |  |  | H | Ladders are only used for light work and not for work that involves carrying heavy or awkward items |  |  |  |  |
| Unsecured ladders |  |  |  |  |  |  |  |  |
|  |  |  | H | Ladders are stored safely and unauthorised use is prohibited |  |  |  |  |
|  |  |  | H | Ladders are set out on a firm base and leaning at the correct angle 75 degrees (1:4)1 |  |  |  |  |
|  |  |  | H | Ladders are tied at the top to a secure structure. If a ladder cannot be tied at the top it is secured at the bottom, or held by a second person |  |  |  |  |
|  |  |  | H | Ladders are not placed on a drum, box or other unstable base |  |  |  |  |
|  |  |  | H | Ladders are removed to storage at the end of each working day to ensure that unauthorised access to roof etc. by others, particularly students, is prevented |  |  |  |  |
|  |  |  | H | Footwear is free from mud/grease before climbing a ladder |  |  |  |  |
|  |  |  | H | 3 points of contact are maintained at the working position |  |  |  |  |

© All Rights Reserved

Cleaning - No.42 Work at Height - Ladders cont’d. (List additional hazards, risks and controls particular to your school using the blank template no.55)

| Hazards | Is the hazard present? Y/N | What is the risk? | Risk rating H = High M=Medium L = Low | Controls (When all controls are in place risk will be reduced) | Is this control in place? | Action/to do list/outstanding controls *Risk rating applies to outstanding controls outlined in this column | Person responsible | Signature and date when action completed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Defective ladders |  | Falls from height | H | The condition of the ladder is checked before each use for cracked or bent stiles or rungs, corrosion, defective or missing fittings or ties. Employees do not use the ladder if it is damaged in any way and report defects immediately to appropriate contact |  |  |  |  |
|  |  |  | H | Wooden ladders should not be painted See Schedule 7 of the Work at Height Regulations |  |  |  |  |
| Overreaching |  | Fall from a height resulting in serious injury or death | H | When using ladders employees do not work on top 3 rungs, or top 2 steps for step-ladders |  |  |  |  |
|  |  |  | H | Where possible an alternative method of working is used, e.g. using extendable poles to clean high level areas |  |  |  |  |
|  |  |  | H | Ladders are properly set up to avoid overreaching |  |  |  |  |
|  |  |  | H | Mobile towers are used where practicable See‘Use Ladders Safely - Information Sheet’available at www.hsa.ie/ publications |  |  |  |  |

84

If there is one or more High Risk (H) actions needed, then the risk of injury could be high and immediate action should be taken. Medium Risk (M) actions should be dealt with as soon as possible. Low Risk (L) actions should be dealt with as soon as practicable.

Risk Assessment carried out by:			Date:	/	/

© All Rights Reserved